Approvals

This document has been reviewed and approved by the following individuals.  By approving this document, each individual acknowledges that it complies with the requirements of the Equipment, Process, and Test Method Validation Procedure (QP 09-002), and all associated procedures.

| Rob Frye Protocol Author |  | Date |  | Denean O’Brien Process Owner |  |  | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| James Catlett Validation Engineer |  | Date |  | Chris Major Quality Assurance |  |  | Date |
| Kenneth Broadley Executive VP |  | Date |  | Paul Amaral CODMAN NEURO Validation Reviewer |  |  | Date |
| Andy Rubino CODMAN NEURO Validation Reviewer |  | Date |  |  |

- PURPOSE

  - The purpose of this Operational Qualification is to qualify that the Kentex Filler K800-MS filling machine for the filling, crimping and lot coding of aluminum tubes that have been filled with n-BCA monomer, BM3044, is properly operating within the intended range.

- SCOPE

  - The scope of this Operational qualification includes verifying cycle parameters on the K800-MS filling machine located in Room #3/B of an ISO Class 8 (Class 100,000) cleanroom at the CMPI manufacturing facility 200 Technology Drive Alpharetta, GA 30005.

  - The scope includes only medical adhesives packaged in aluminum tubes.

    - The medical adhesive filled on the Kentex K800-MS machine will be substituted during this qualification with a chemically similar, nBCA based material and filled into aluminum tubes with dimensions of approximately 10 mm + 0.35mm in diameter and 45 mm + 1 mm in body length which are purchased by CMI from an approved supplier.

  - This is the initial validation for this equipment.

- DEFINITIONS

  - Acceptance Criteria – A set of pre-determined requirements upon which pass/fail decisions are based. Acceptance criteria must be specific and measurable.

  - Installation Qualification (IQ) – Establishing by objective evidence that all key aspects of the process equipment and ancillary system installation adhere to the manufacturer’s approved specification and the recommendations of the equipment supplier are suitably noted.

  - Ppk – Process Capability; an estimate of the process capability of a process during its initial set-up before it has been brought into a state of statistical control

  - Process Validation – Establishing by objective evidence that a process consistently produces a result of a product meeting its predetermined requirements.

  - Protocol – A pre-approved document which states the requirements and procedures for conducting a validation, including test parameters, product characteristics, manufacturing equipment, and decision points for acceptance.

  - Verification – Confirmation through documentation and objective evidence and/or testing, that a specified requirement has been met.

- REFERENCES

  - K800-MS Machine Instruction Manual.

  - Validation Protocol 631VAP0014

  - VMP 159 Validation Master Plan for Kentex Filler-Crimper

  - VAP 159-1 Installation Qualification for Kentex K800-MS Filler-Crimper

  - QP 09-002 Test Method Validation Procedure

  - VAP 010 Installation and Operational Qualification of the K-800 MS Tube Crimper

  - MP 09-041 Kentex K800-MS Filling Procedure

  - MP 09-112 Cleanroom Gowning Procedure

  - MS-ES02315001 Material Specification for nBCA

- RESPONSIBILITIES

  - Validation Team Leader is responsible for the development of the OQ protocol, the accurate translation of the protocol into executable procedures and the subsequent analysis of the OQ data.  The Validation Team Leader is also responsible for carrying out the validation per the protocol and noting any procedural deviations that occur throughout the process.

  - Validation Engineer is responsible for assisting in protocol development and the executing of the OQ protocol.

  - Validation Approver is a member from the Codman Team and is responsible for assisting with the completion of validation activities.  He/She reviews and approves the validation protocols and report.

  - Executive VP oversees the validation to ensure that the parameters of the protocol are sufficient to validate the system.  He reviews and approves the validation protocols and report.

  - Validation Team:

| Function | Name |
| --- | --- |
| Protocol Author | Rob Frye |
| Validation Reviewer (Codman) | Paul Amaral |
| Validation Reviewer (Codman) | Andy Rubino |
| Validation Engineer | James Catlett |
| Validation Team Leader | Denean O’Brien |
| Executive VP | Kenneth Broadley |
| QA Manager | Chris Major |

- SAFETY CONSIDERATIONS

  - Disconnect the machine from its power source before cleaning or reaching into the machine.

  - Use of a plug socket is not advisable.  A non-fuse switch is recommended for this purpose.

  - Do not place any objects on the working platform to avoid any dangers when in operation.

  - Any hand contact is forbidden when the machine is running in operation.

  - Do not open the electrical control box when machine is in operation to avoid electrocution.

  - Unqualified personnel should avoid speed adjustments.

  - Do not place tools on top of the machine to avoid complications during operation.

  - PPE Requirements

    - There are no additional PPE requirements beyond standard Clean Room apparel and gowning requirements.

- OQ DOCUMENTATION REQUIREMENTS

  - Data will be collected on the data sheets provided in this protocol.  Raw data may be summarized in the final report; however, the raw data or reference to the raw data location will be included in the final report.

  - Specific document requirements are specified in the checklists in the appendix.

- ENVIRONMENTAL REQUIREMENTS

  - The Operational Qualification shall be conducted in Room #3/B of an ISO Class 8 (Class 100,000) cleanroom at the CMI manufacturing facility located at 200 Technology Drive Alpharetta, GA 30005.

- EQUIPMENT DESCRIPTION

  - The following equipment qualified in VAP159-1 Installation Qualification of Kentex Filler-Crimper will be used to conduct this OQ.  In addition, any approved gaging or approved materials may be used and documented in the Data Collection sheets.

    - Kentex K800-MS, Serial No. T0880

- PROCEDURE

  - Materials

    - The materials used in the Operational Qualification to mimic n-BCA adhesive.

Table 1 – Materials Used in OQ

| Material | Representative Name/Description |  | Part Number |  | Approximate Value | Codman Specification |
| --- | --- | --- | --- | --- | --- | --- |
| Adhesive | nBCA monomer |  | BM3044 |  | 3 – 4 cp | Range: ≤10 |
| Tubes | Pre-Inspected Codman Tubes |  | 14550013 |  | Quantity: Record Value | Lot no.: Record value |

  - Lot Size Definition – Each run will be a minimum of 1000 filled tubes, equal to approximately one third of a production run.

  - Process Parameters

    - Critical To Quality (CTQ) – The following response variables will be inspected as CTQ and are part of the Criteria for Success.

      - Visible Leak – Attribute Measurement

      - Tube Graphic or tube defects- Attribute Measurement

    - Critical Machine Settings are input factors which describe the operating window being tested.  The critical settings are shown in three test runs that define an operating window of worst case conditions with a nominal condition run.

    - All data will be recorded in the data sheets provided in the appendix.

Table 2 – Settings Used in OQ

This table defines the target values for each level.  Levels will be set as close as is practical.

| Factor | Level |  | Set Point Value |  |
| --- | --- | --- | --- | --- |
| Crimp Height | Nominal |  | 4.5 mm (±0.5) |  |
| Crimp Thickness | Nominal |  | ≤1.25mm |  |
| Dial Speed | Low |  | 20 cycles/min |  |
| Dial Speed | Nominal |  | 30 cycles/min |  |
| Dial Speed | High |  | 40 cycles/min |  |
| Fill Weight | Nominal |  | 1 g (+ 0.05g/ - 0.0g) |  |

  - Testing to be Performed

    - The qualification runs may be run in any order.

    - Set-up data will be recorded prior to the start.

    - Testing will be performed by the Engineer and personnel trained to this OQ and Kentex (K800-MS) Filling Procedure.

    - Collect and label finished tubes for testing and remove from room to inspection area.

    - Perform line clearance and remove all materials from the room.

    - Clean and purge the machine.

    - Data for each run will be collected on the data sheets.

    - Upon completion of all runs, the OQ will be complete.

    - If a run cannot be completed due to an unforeseen event, the cause of the termination of the run will be recorded and the run may be rerun.

  - Material Disposition

    - Material will be measured and later destroyed using an approved method.

- ACCEPTANCE CRITERIA

  - The statistical criteria for attribute characteristics are 95% reliability at 95% confidence using a minimum sample size of 278 and acceptance criteria of accept on zero and reject on 1.

  - The statistical criteria for variable characteristics are 1.0 PpK capability using a minimum sample size of 32.

  - If Ppk is 1.0, the system is producing 99.73% of its output within specifications. The larger the Ppk, the less the variation between process output and specifications.

  - All requirements in all checklists must pass for the OQ Data Collection to Pass, but does include approved deviations.  All requirements for data collection for the OQ test runs are contained in the checklists.

  - Finished Product Specifications

    - Finished product from each run will be inspected according to the following criteria and must pass for the OQ to Pass.

Pre-Sterile Criteria for Success

| Critical Product Characteristics | Minimum Sample Size per Run | Specification | Pass / Fail Criteria |
| --- | --- | --- | --- |
| Fill Weight | 32 | Short term capability of 1.0 PpK or higher. | Pass: ≥ 1.0 PpK Fail: < 1.0 PpK |
| Visible Leaks | 32 | No visible adhesive leak from tube. | None - For Data Collection only |
| Graphic/Tube defects | 32 | No defects on Graphics as outlined in Inspection Criteria MS-ES02315001 | Pass - Zero defects. Fail - 1 or more defects. |
| Lot Number | 32 | Lot number is legible. | Pass - Zero defects. Fail - 1 or more defects. |

Pre-Sterile - For Data Collection and Evaluation Only

| Critical Product Characteristics | Minimum Sample Size per Run | Specification | Pass / Fail Criteria |
| --- | --- | --- | --- |
| Crimp Thickness | 32 | Meets current specification | Pass/Fail |
| Crimp Height | 32 | Meets current specification | Pass/Fail |

- DEVIATIONS

  - All deviations shall be approved and documented in the final report including rationale.

- APPENDICIES

  - Training Verification

  - Pre-fill Check

  - OQ Run Data Sheets

    - Data Sheets and Results for RUNS

APPENDICIES

By signing below, each individual acknowledges that he/she has been trained and understands both the contents of this Operational Qualification (VAP 159-2) as well as their role in the process validation that will be conducted.

TRAINING VERIFICATION

| Print Name | Signature | Date |
| --- | --- | --- |

Pre-fill Check

| RUN NUMBER (circle run number) |  | Operator(s) Initial/date |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1/2/3 |  |  |  |  |  |  |  |
| SCALE ID: | CAL DUE: | CALIPER ID: |  |  | CAL DUE: |  |  |
| Kentex Set-up |  |  |  |  |  |  |  |
| Dry Fill Check Inspect 16 empty tubes for label appearance, crimp width, crimp thickness and lot stamp legibility. Check “yes” if they meet specifications. |  |  | Label Appearance | Crimp Width (4-5 mm) |  | Crimp Thickness <1.25mm | Lot# legible |
|  |  |  | ☐Yes ☐ No | ☐Yes ☐ No |  | ☐Yes ☐ No | ☐Yes ☐ No |

| Pre-fill Check | Tube | Label Appearance | crimp height 4-5 mm | crimp thickness <1.25mm |  | Lot # Legible | Empty Tube Wt. | Filled Tube Wt. | Wt. of adhesive in tube |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check 16 filled tubes before beginning filling operations. Weigh and record the weight of an empty tube before filling. After filling the tube, weigh it again and determine how much adhesive is in the tube. 1.00g + 0.05g – 0.0g. Inspect for label appearance, the width of the crimp, the thickness of the crimp, and lot number stamp legibility. Where applicable, check the box to indicate satisfactory results. | 1 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 2 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 3 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 4 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 5 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 6 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 7 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 8 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 9 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 10 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 11 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 12 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 13 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 14 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 15 | ☐ |  |  |  | ☐ |  |  |  |  |
|  | 16 | ☐ |  |  |  | ☐ |  |  |  |  |
| Performed by/date: |  |  |  |  | Checked by/date: |  |  |  |  |

Data Sheets for OQ RUN #

| Instruction | Verification |
| --- | --- |
| - Line Clearance Completed; Verify complete and initial complete. | Initials |
| - Adhesive Used– Record Part Number and Lot Number. | PN – Lot No. - |
| - Tubes Used – Record Part Number and Lot Number. | PN – Lot No. - |
| Machine Speed Setting; Set machine Speed to 20 Cycles/Minute and initial complete. | Initials |
| - Set fill setting for Fill. Fill 16 samples and record weight. | Initials |
| - Finished Tube Lot Number Used for OQ Run | Lot No. - |

Data Sheet Completed By ______________________________________ Date __________________

Approved By ______________________________________ Date __________________

Proceed to Run # ______________ (Initial/date)

Data Sheet:

| Run #____ | Initials / Date: |  | Units per Minute | 20/30/40 |
| --- | --- | --- | --- | --- |

  - Each run will be a minimum of 1000 filled tubes.

  - Select and number 32 empty tubes. Document empty tube weight below.

  - Place numbered tubes randomly into the feeder throughout the run. Measure and document data below.

| Sample No. | Filled Tube Weight (g) | Empty Tube Weight (g) | Net Adhesive Fill Weight (Calculate g) | Tube Crimp Height (mm) | Tube Crimp Thickness (mm) |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 9 |  |  |  |  |  |
| 10 |  |  |  |  |  |
| 11 |  |  |  |  |  |
| 12 |  |  |  |  |  |
| 13 |  |  |  |  |  |
| 14 |  |  |  |  |  |
| 15 |  |  |  |  |  |
| 16 |  |  |  |  |  |
| 17 |  |  |  |  |  |
| 18 |  |  |  |  |  |
| 19 |  |  |  |  |  |
| 20 |  |  |  |  |  |
| 21 |  |  |  |  |  |
| 22 |  |  |  |  |  |
| 23 |  |  |  |  |  |
| 24 |  |  |  |  |  |
| 25 |  |  |  |  |  |
| 26 |  |  |  |  |  |
| 27 |  |  |  |  |  |
| 28 |  |  |  |  |  |
| 29 |  |  |  |  |  |
| 30 |  |  |  |  |  |
| 31 |  |  |  |  |  |
| 32 |  |  |  |  |  |
| PpK Calc. > |  |  |  | For Evaluation Only. | For Evaluation Only. |
| Result > |  |  | Pass / Fail |  |  |

Attribute Measurements Data

| Attribute | Sample Size | Specification | Pass/Fail Criteria | Result |
| --- | --- | --- | --- | --- |
| Lot Number | 278 | Lot Number is Legible | Pass - Zero defects. Fail - 1 or more defects. | Pass / Fail |
| Tube Crimp Thickness | 278 | ≤1.25mm | None – Data collection only. | For Evaluation Only |
| Tube Crimp Height | 278 | 4.5 mm (±0.5) | None – Data collection only. | For Evaluation Only |
| Visible Leaks | 278 | No visible leaks. | Collect data. | For Evaluation Only |
| Graphics/tube defects | 278 | No defects on Graphics as outlined in Inspection Criteria, MS-ES02315001 | Pass - Zero defects. Fail - 1 or more defects. | Pass / Fail |

OQ Run #              Pass / Fail

Data Sheet Completed By ______________________________________ Date __________________

Data Sheet Verified By ________________________________________ Date __________________

QA Approved By ______________________________________________ Date __________________