- PURPOSE

  - This protocol outlines the Operational Qualification (OQ) for the Autoclaved Items Labeling Process using PRT015, the ZebraDesigner software, the Steri-ThermoTagTM – AUTT label stock and a Thermal Transfer resin ribbon, S-18381.

Figure 1: PRT015 (Zebra ZT421), Steri-ThermoTAGTM – AUTT label, and Thermal Transfer Resin Ribbon

- SCOPE

  - The OQ includes evaluation of operational ranges of the printer and the process variables and their impact to Critical to Quality (CTQ) attributes:

    - Legibility of the printed labels.

    - Adhesiveness of the labels to the autoclaved item pouches and wraps, the critical-to-quality attributes, of the printed Autoclaved items labels.

    - Accuracy of the information (e.g., sterilization load number) printed on labels  are verified in-process and will not be evaluated in this PQ.

  - Storage environmental conditions of the blank and printed labels will not be evaluated because of the operating range of the labels are considerably wider than the temperature specifications of the warehouses, Support Labs and Cleanrooms where the blank and printed labels are stored.

| Item/Area | Operating Range |
| --- | --- |
| Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels | -60°C to +150°C (-76°F to + 300°F) & Autoclavable up to 20 times @ 121°C |
| Warehouse | 15°C to 30°C (59°F to 102.2°F) |
| Support Labs | 15°C to 39°C (59°F to 86°F) |
| Clean rooms | 7°C to 29.5°C (44.6°F to 85.1°F) |

  - Material variability will not be included in the OQ as new materials (i.e., new label stock and new resin ribbons) will be added to the existing incoming inspection process with new inspection forms (IAS) created.

- BACKGROUND

  - Autoclaved Items Labeling Process

    - Recent discoveries revealed that the current label stocks used in the Autoclaved Items Labeling Process are not suitable for the application.  A new label stock, Steri-ThermoTAGTM – AUTT, designed specifically for steam sterilization has been identified.  The labels will be thermal transfer printed using a Zebra printer, PRT015, with a resin ribbon suitable for the printer.

    - Item description labels will be affixed to pouched or wrapped items prior to steam sterilization.  Autoclave load ID labels with corresponding sterilization load ID, initials and date of technician labeling the items will be affixed to the wrapped or pouched items immediately prior to or following steam sterilization once items are cooled enough to be handled.

    - New documents will be created and affected documents will be revised to reflect the updated Autoclaved items labeling process.

    - The new/revised documents will be finalized, based on the results of the validation, released, and associated training completed as part of PCR22-021.

  - PRT015 (ZT421 Zebra Printer)

    - was purchased as an off the shelf, bench top unit with a metal housing cover to protect the internal components.

    - is capable of direct thermal printing and thermal transfer printing (with ribbon).   The 203-dpi version prints up to 12”/second on up to 6” wide labels.  It is equipped with a 4.3” color touch display with Serial, USB, Ethernet and Bluetooth® interface, 3” media supply hanger, an auto calibration when the printer is turned on or when print head is closed and  auto-switching 100-240V power supply features.  The ZT421printer is 12.75” or 324 mm (H) x 13.25” or 336 mm (W) x 19.50” or 495mm (D) and weights 40 lb or 18.14 Kg.

    - does not require calibration.

  - Permanent Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels are high temperature resistant permanent labels for the identification of various surfaces that undergo autoclaving and dry heat sterilization.  Temperature ranges are from -60°C to +150°C (-76°F to + 300°F).  The labels can be applied to various surfaces and withstand 20 Autoclave sterilization cycles at 121°C (250°F) and 16.5psi pressure.  The labels are waterproof, weatherproof and thermal-transfer printable and are available in various sizes and colors.

  - Resin thermal transfer ribbons are waterproof, grease, chemical and solvent resistant resin ribbons for use with polyester and synthetic labels.  The resin ribbons are available in various sizes.

  - The ZebraDesigner software allows users to customize and create labels in a variety of types and sizes and print them to the Zebra printer.

  - Proposed autoclaved items labeling process steps:

    - Item Description labels and Sterilization Load ID labels are printed using the same label stock and using the same printing technology (thermal-transfer).

    - The OQ will include affixing Sterilization Load ID labels to the wrap/pouch materials prior to and following sterilization to allow process flexibility.

- REFERENCES

  - FDA Guidance for Industry – Process Validation: General Principles and Practices

  - American Association for Tissue Banks Standards for Tissue Banking

  - Food and Drug Administration 21 CFR Part 1271

  - GHTF/SG3/N99-10:2004 Quality Management Systems – Process Validation Guidance

  - VAP22-004-00 Autoclaved Items Labeling and Zebra Printer (PRT015 and PRT016) Process Validation Plan

  - QSP019-001 Process Changes

  - QSP035-001 Risk Management

  - FRM011-003 Group Training Form

  - FRM015-007 Instrument Sterilization Form

  - SOP013-001 Master Safety Plan

  - SOP012-001 Master Validation Plan

  - SOP012-002 Definitions, Abbreviations, and Acronyms Used by StimLabs

  - SOP015-003 Operation of Steris Lab Sterilizer

  - SOP015-008 Review and Handling of Chart Recorder Data

  - SOP020-001 Master Validation Plan

  - WI014-002 Preparation and schedule of Disinfectants

  - WI015-003 Decontamination and Sterilization of Instruments

  - WI015-004 Instrument Preparation for Sterilization

  - WI015-017 Operation of the Zebra Printer

  - Steri-ThermoTAGTM Label Product Data Sheet

  - Steri-ThermoTAGTM Label Product Catalog

  - ULINE Product Catalog

  - VAR18-001-02 Zebra Printer PRT001 IQ report

  - ZT411/ZT421 Industrial Printer User Guide

  - ZT421 Technical Specifications

  - Zebra ZT411/421 Regulatory Guide

  - ZT411, ZT421 Set up Guide

  - ZT600 Series User Guide

  - ZT600 Series Technical Specifications

  - ZT600 Series Product Spec Sheet

  - PCR22-021 Autoclaved Items Labeling Process

- DEFINITIONS

  - 70% Isopropyl Alcohol (IPA/70% IPA): Isopropyl Alcohol, Concentration 70%

  - Acceptance Criteria:  Pre-determined requirements upon which pass/fail decisions are based.  Acceptance criteria are specific and measurable.

  - Ancillary Area (Support Labs): Areas that are used for support of the cleanrooms. Areas may include sinks and other equipment for cleaning, disinfection and autoclaving of instruments and materials.

  - Autoclave (Sterilizer): Equipment using steam and high heat levels to eliminate or remove all forms of life on an object’s surface such as bacteria, fungi, viruses, and spores, with the exception of prions.

  - Autoclave Pouch: Self-sealing pouches with a chevron seal designed to withstand sterilization.

  - Autoclave Wrap: Low-lint fabric that is designed to withstand sterilization.

  - Chart Recorder: Equipment used to monitor temperature and/or humidity in a specific area by recording information onto a paper disc or paper strip.

  - Critical to Quality:  Attribute of a part, assembly, sub-assembly, product or process that has direct and significant impact on its actual or perceived quality to the customer or end user.

  - Data Logger: An electronic device used to record environmental data over time within a piece of equipment or in a specific location via external instruments and sensors.

  - Deviation:  The action of departing from an established method, procedure, or acceptance criteria.

  - Disinfectant:  An agent (e.g., heat or a chemical) capable of reducing the number of viable microorganisms. A disinfectant might not kill spores. Use of antimicrobials in tissue processing is included here.

  - First Expired, First Out (FEFO): FEFO is an inventory management policy where the tissue product expiring closest to the current date is also the first tissue product that is removed from inventory.  This policy ensures tissue products close to expiry are used first.

  - First In, First Out (FIFO): FIFO is an inventory management policy where the first item that is logged into inventory is also the first item that is removed from inventory.  This policy ensures proper stock rotation.

  - GxP: Generic abbreviation to encompass the current Good Laboratory Practices (cGLP), current Good Manufacturing Practices (cGMP), and current Good Clinical Practices (cGCP). GxP can refer to one specific set or to any combination of the three.

  - IAS: Incoming Acceptance Specification

  - LSL:  Lower Specification Limit; used to determine process capability.

  - Normal Distribution:  A function that represents the distribution of many random variables as a symmetrical, bell-shaped graph.

  - Operational Qualification:  Establishing by objective evidence the process control limits and action levels which result in a product that meets predetermined requirements.

  - PpK/CpK:  Process capability; an estimate of the process capability of a process during its initial set-up before it has been brought into a state of statistical control.

  - Process Validation:  Establishing by objective evidence that the output of a process consistently meets predetermined specification.

  - Protocol:  A pre-approved and prospective document that states the requirements and procedures for conducting a validation.

  - USL:  Upper specification limit; used in determining process capability.

  - Validation/Study Leader: The individual who has the responsibility and authority over the operation of the equipment or process being validated.

  - Verification:  Confirmation by examination with a provision of documented evidence that a specific requirement has been fulfilled.

- RESPONSIBILITIES

| Function | Name | Title | Author Protocol & Report | Review and Approve Protocol & Report |
| --- | --- | --- | --- | --- |
| Validation/Study Leader | Sasi Songthong | Senior Quality Engineer | X |  |
| MwER/MR/HOQ/TBD | Melissa O’ Connor | Chief Quality & Regulatory Officer |  | X |
| Operations Management | Joel Cummings | Vice President of Operations |  | X |
| Quality Management | Justin Junnier | Continuous Improvement Manager |  | X |

- ENVIRONMENTAL REQUIREMENTS

  - Temperature: 40°F - 104°F (5°C - 40°C)

  - Humidity: 20% - 85% non-condensing R.H.

- SAFETY CONSIDERATIONS

  - Standard office and laboratory safety hazards that may be encountered during this validation will be addressed according to StimLabs Master Safety Plan.

  - Refer to Operation of Steris Lab Sterilizer work instructions for additional safety precautions and PPE.

- PROCEDURE

  - Printer Descriptions

| Equipment I.D. | Manufacturer | Model # | Serial # | Printer Location |
| --- | --- | --- | --- | --- |
| PRT015 | Zebra | ZT421 | 99N220801129 | Support Lab 1 |

  - Raw Materials

    - Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels, P/N AUTT-19C3-1WH

      - The Steri-ThermoTAGTM labels are available in various sizes from 1” x 0.6875” to 4” x 2”.

      - AUTT-19-C3-1WH labels (1.3” x 0.625” (33 mm x 15.9 mm)), one of the smallest label sizes available of this label stock, will be utilized during the OQ as smaller labels are more challenging for the end users to read (legibility) and have smaller area of the adhesive backing (adhesiveness), compared to larger labels.

    - ULINE Industrial Thermal Transfer Ribbons – Resin, 2.36” x 1,476’. Model No. S-18381

      - Resin ribbons are available in various sizes.  The S-18381 was selected as it is compatible with Zebra printer and AUTT-19 labels.

    - IAS16-034 Autoclave Pouches

    - IAS16-029 70% IPA

    - IAS16-047 Spor-Klenz 3200 mL Ready-to-Use (RTU) Bottles

    - IAS17-036 Sequential Sterilization Wrap or IAS21-003 Simultaneous Sterilization Wrap

      - Either Sequential or Simultaneous Sterilization Wrap type is acceptable for the OQ as they are made from the same materials.

    - IAS21-001 Vesphene III Concentrate Pack, prepared

    - Twenty-Four (24) items to be pouched and autoclaved

    - Twenty-Four (24) items to be wrapped and autoclaved

  - Critical to Quality (CTQ) Attributes

    - Legibility of the printed labels: labels are to remain legible from the time they are printed until the labels, along with packaging materials, to which they are affixed, are discarded.

    - Adhesiveness of the labels: labels are to adhere to the packaging materials of the Autoclaved items from when they are affixed to the packaging materials until the packaging materials are discarded.

  - Process Variables

    - Critical Printer Settings:

      - Print Speed settings of 4-6, previously determined to be acceptable operational settings, have been historically used in other Zebra printers’ qualifications at StimLabs and will be used during this qualification.   See Zebra Printer PRT001 IQ report for an example of a previous qualification using Print Speed settings range of 4-6.

      - Print Darkness settings of 23 – 27, previously determined to be acceptable operational settings, have been historically used in other Zebra printers’ qualifications at StimLabs and will be used during this qualification.  See Zebra Printer PRT001 IQ report for an example of a previous qualification using Print Darkness settings range of 23-27.

    - Design of Experiments/Process Variables Matrix

      - Printer Settings

        - The OQ will incorporate the operational limits of the print speed and print darkness matrix in the operational range.

| Operational Range Matrix |  |  |  |
| --- | --- | --- | --- |
| Settings | Speed 4 | Speed 5 | Speed 6 |
| Darkness 23 | Low/Low |  | High/ Low |
| Darkness 24 |  |  |  |
| Darkness 25 |  |  |  |
| Darkness 26 |  |  |  |
| Darkness 27 | Low/ High |  | High/ High |

        - Four (4) sets of printer settings will be utilized to print the labels for this OQ.

          - Low/Low: Print Speed 4 and Darkness 23

          - Low/High: Print Speed 4 and Darkness 27

          - High/Low: Print Speed 6 and Darkness 23

          - High/High: Print Speed 6 and Darkness 27

      - Autoclave Cycle Types:

        - The OQ will incorporate the 2 Autoclave cycle types utilized by StimLabs:

          - Gravity used in AUT001

          - Pre-Vacuum used in AUT002

      - Autoclaved Items Package Type:

        - Autoclave Pouch

        - Sterilization Wrap

      - Labeling:

        - Prior to sterilization

        - Following sterilization

      - Handling of labeled pouched and wrapped items during inventory will be simulated by:

        - performing the OQ in the Support Lab 1, where labeled packaged instruments are typically stored prior to and following sterilization.

        - handling each labeled affixed to an item with

          - Dry gloved hands

          - Gloved hands, which have been wiped with isopropyl alcohol (IPA)

          - Gloved hands, which have been wiped with Vesphene III

          - Gloved hands, which have been wiped with Spor-Klenz..

        - NOTE: IPA, Vesphene III, and Spor-Klenz are common disinfectants used in the production environment.

        - shuffling the labeled pouched and wrapped items, pre and post autoclaving, to simulate First In, First Out (FIFO)/ First Expired, First Out (FEFO) inventory management policy utilized in the production environment at StimLabs.

      - Personnel performing the tasks:

        - A minimum of 3 operators will be utilized for:

          - Inspection of printed labels for legibility and adhesiveness

          - Affixing the printed labels to the pouched or wrapped items

            - Prior to an Autoclave run

            - After an Autoclave run

          - Simulated handling

    - Test Equipment

      - Temperature and humidity recorders (e.g., a chart recorder or a data logger)

    - Test Methods

      - Simulated handling of items

      - Visual inspection for legibility of labels

      - Visual inspection for presence of labels after simulated handling and storage of items.

    - Performing the OQ

      - Follow each step outlined in the OQ Scheme (Figure 1) and document results in the corresponding data collection sheet in the APPENDIX.

        - Detailed instructions are provided on each data collection sheet.

Figure 1: OQ Scheme

NOTE:

Set 2 and Set 3 Labels represent Sterilization Load ID Labels to be affixed immediately prior to and after sterilization, respectively.

NOTE: Set 1 Labels

represents Item Description Labels

- LOT SIZE & SAMPLING PLAN

  - Lot Size

    - Each sterilization load contains 100 or fewer pouched or wrapped packages.

    - Each Autoclaved package contains 2 labels – an item description label and a sterilization load ID label.

    - Labeled autoclaved items are inspected in process during production.

  - Sampling Plan & Statistical Rationale

    - A total of 144 labels will be printed and evaluated during the OQ using 4 sets of printer settings, which reflect the defined operational range.

| Group | Printer Settings | Print Speed | Print Darkness | Label Number |
| --- | --- | --- | --- | --- |
| 1 | Low/Low | 4 | 23 | 1 - 12 |
| 2 | Low/High | 4 | 27 | 13- 24 |
| 3 | High/Low | 6 | 23 | 25-36 |
| 4 | High/High | 6 | 27 | 37-48 |
| 5 | Low/Low | 4 | 23 | 49-60 |
| 6 | Low/High | 4 | 27 | 61-72 |
| 7 | High/Low | 6 | 23 | 73-84 |
| 8 | High/High | 6 | 27 | 85-96 |
| 9 | Low/Low | 4 | 23 | 97-108 |
| 10 | Low/High | 4 | 27 | 109-120 |
| 11 | High/Low | 6 | 23 | 121-132 |
| 12 | High/High | 6 | 27 | 133-144 |

    - Groups of 12 labels will be evaluated under multiple process variables, categorized into 12 process scenarios, to which labels will be exposed in the production environment.

| Line # | Process Scenarios for OQ |
| --- | --- |
| A | Label (On Pouch), Inventory, Autoclave (AUT001 Gravity), Inventory |
| B | Label (On Wrap), Inventory, Autoclave (AUT001 Gravity), Inventory |
| C | Label (On Pouch), Inventory, Autoclave (AUT002 Pre-Vac), Inventory |
| D | Label (On Wrap), Inventory, Autoclave (AUT002 Pre-Vac), Inventory |
| E | Inventory, Label (On Pouch), Autoclave (AUT001 Gravity), Inventory |
| F | Inventory, Label (On Wrap), Autoclave (AUT001 Gravity), Inventory |
| G | Inventory, Label (On Pouch), Autoclave (AUT002 Pre-Vac), Inventory |
| H | Inventory, Label (On Wrap), Autoclave (AUT002 Pre-Vac), Inventory |
| I | Inventory, Autoclave (AUT001 Gravity), Label (On Pouch), Inventory |
| J | Inventory, Autoclave (AUT001 Gravity), Label (On Wrap), Inventory |
| K | Inventory, Autoclave (AUT002 Pre-Vac ), Label (On Pouch), Inventory |
| L | Inventory, Autoclave (AUT002 Pre-Vac), Label (On Wrap), Inventory |

    - Sampling Plan & Statistical Rationale

      - The sample size of 11 labels (90%/80% assurance level) has been identified based on the low risk region, determined by occurrence, severity and detection ratings and sample size guidance in the Risk Management procedure.

        - The likelihood (occurrence) for illegible or detached Autoclaved items labels impacting products sold is rare due to the 100% in-process verification in place.

        - The severity of the illegible or detached Autoclaved items labels is minor as the impact can be readily absorbed but some management effort is required.  Some GXP data may be lost.

        - Because of 100% in-process verification, the likelihood of detection is very high.

      - To ensure the number of samples are evenly distributed within the OQ scheme (Figure 1), 12 labels, instead of the 11 labels identified in 10.2.3.1, for each of the process scenarios (10.2.2) will be utilized for the OQ.

        - All 4 sets of printer settings, Low/Low, Low/High, High/Low and High/High, will be incorporated in each process scenarios.

- ACCEPTANCE CRITERIA

  - All 144 labels are legible after undergoing simulated handling.

  - All 144 labels adhere to each Pouches/Wraps after undergoing simulated handling.

- MATERIAL DISPOSITION

  - Upon a successful OQ completion with passing results, the Permanent Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels sized 1.3” x 0.625” and larger and the corresponding sized thermal transfer resin ribbons will be deemed appropriate for the autoclaved items labeling process in the production environment, if the sizes are compatible with the Zebra printer used.

- DOCUMENTATION REQUIREMENTS

  - Results shall be summarized in an OQ Report.

  - Various Control Plans, including validated process settings and the re-validation criteria shall be documented in the final report.

  - Raw data sheets shall be included in the appendix of the OQ report and/or a reference to the location of the raw data shall be included in the OQ report.

  - Calibration records of the chart recorder(s)/data logger(s) shall be attached to the OQ report (in the Appendix).

  - Signed OQ Protocol Training forms will be attached to the OQ report (in the Appendix).

- DEVIATIONS

  - Deviations to the OQ will be pre-approved using FRM020-012 Protocol Deviation Form and explained in the OQ report.

  - Copies of signed Protocol Deviation Forms will be attached to the OQ report (in the Appendix).

- PRE-REQUISITE STEPS

  - Approved Process Validation Plan (PVP)

  - IQ results must be quality reviewed before validation activities may begin.

  - Approval and release of raw materials

  - Verification of environmental requirements

  - Completed Operator Training

  - Draft work instructions

  - Calibration records of the chart recorder(s)/data logger(s)

  - The chart recorder(s)/data logger(s) are set up and started recording temperature and humidity reading in Support Lab 1 prior to starting the OQ.

  - The OQ Protocol must be signed before validation activities may begin.

- APPENDIX

  - Data Collection Sheet: TRAINING FORM EXAMPLE

  - Data Collection Sheet: PRE-REQUISITE CHECKLIST

  - Data Collection Sheet: LABEL PRINTING – SET 1

  - Data Collection Sheet: AUTOCLAVED ITEMS PREPARATION

  - Data Collection Sheet: SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION

  - Data Collection Sheet: LABEL PRINTING – SET 2

  - Data Collection Sheet: LABELING BEFORE STERILIZATION

  - Data Collection Sheet: STERILIZATION

  - Data Collection Sheet: LABEL PRINTING – SET 3

  - Data Collection Sheet: LABELING AFTER STERILIZATION

  - Data Collection Sheet: SIMULATED HANDLING & INSPECTION AFTER STERILIZATION

APPENDIX 16.1 TRAINING FORM EXAMPLE

EXAMPLE

Instructions: Use current revision of the form available in MasterControl.

APPENDIX 16.2 PRE-REQUISITE CHECKLIST

| Pre-Requisite Activities |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Instructions: 1. Verify each line item and circle the one of the bolded fonts for actual results. 2. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 3. Record comments and additional information in the “Comments” section or N/A. 4. Passing results are required to continue with the OQ. |  |  |  |  |
| Line # | Expected Results |  | Actual Results | Pass (P) / Fail (F) |
| 1 | Process Validation Plan (PVP) is approved |  | Process Validation Plan (PVP) is / is not approved |  |
| 2 | IQ results have been quality reviewed |  | IQ results have / have not been quality reviewed |  |
| 3 | Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels have been inspected & released for use |  | Steri-ThermoTAGTM – Autoclave-Resistant Thermal-Transfer Labels have / have not been inspected and released for use |  |
| 4 | Resin ribbon has been inspected and released for use |  | Resin ribbon has / has not been inspected and released for use |  |
| 5 | Environmental requirements have been verified |  | Environmental requirements have / have not been verified |  |
| 6 | Work instructions have been drafted |  | Work instructions have / have not been drafted |  |
| 7 | Calibration records for the chart recorder(s)/data logger(s) are available |  | Calibration records for the chart recorder(s)/data logger(s) are / are not available |  |
| 8 | The chart recorder(s)/data logger(s) are set up and started recording temperature and humidity reading in Support Lab 1 |  | The chart recorder(s)/data logger(s) is / is not set up and started / has not started recording temperature and humidity reading in Support Lab 1 Equipment ID:_________________________ Calibration Due:_______________________ |  |
| 9 | OQ Protocol is approved |  | OQ Protocol is / is not approved. |  |
| 10 | All pre-requisite activities (Lines 1 – 9) are complete |  | All pre-requisite activities (Lines 1 – 9) are / are not complete |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.3 LABEL PRINTING – SET 1

| Group 1 Low/Low Printer Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP1. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#1 – Label#12) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels. |  |
| 4 |  | Each of the 12 labels contains a unique label number (#1 - #12). | Each of the 12 labels contains / does not contain a unique label number (#1 - #12). |  |
| 5 | Legibility | All labels are legible | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.3 LABEL PRINTING – SET 1 (Cont.)

| Group 2 Low/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP2. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#13 – Label#24) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels. |  |
| 4 |  | Each of the 12 labels contains a unique label number (#13 - #24). | Each of the 12 labels contains / does not contain a unique label number (#13 - #24). |  |
| 5 | Legibility | All labels are legible | All labels are / are not legible |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.3 LABEL PRINTING – SET 1 (Cont.)

| Group 3- High/Low Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP3. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#25 – Label#36) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#25 - #36). | Each of the 12 labels contains / does not contain a unique label number (#25 - #36). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.3 LABEL PRINTING – SET 1 (Cont.)

| Group 4 - High/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP4. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#37 – Label#48) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#37 - #48). | Each of the 12 labels contains / does not contain a unique label number (#37 - #48). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.4 AUTOCLAVED ITEMS PREPARATION

| Pouched Items |  |  |  |
| --- | --- | --- | --- |
| Instructions: 1. Each of the 3 operators prepares 8 pouched items to be autoclaved according to standard procedure, labels each item as specified in the table below, and document the completion. 1.1 Select the location on the plastic side near the seal flap to ensure the label does not obstruct Information (e.g., serial number, quantity, or lot number, etc.) on the instrument or on the pouch. 1.2 Using a thumb, affix the label onto the pouch. 1.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 2. Record comments and additional information in the “Comments” section or N/A. 3. Set the labeled pouched items aside upon completion. |  |  |  |
| Instrument Description |  | Label # | Items Prepared and Labeled By (initials and date): |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
|  |  | 13 |  |
|  |  | 14 |  |
|  |  | 15 |  |
|  |  | 25 |  |
|  |  | 26 |  |
|  |  | 27 |  |
|  |  | 37 |  |
|  |  | 38 |  |
|  |  | 39 |  |
|  |  | 4 |  |
|  |  | 5 |  |
|  |  | 6 |  |
|  |  | 16 |  |
|  |  | 17 |  |
|  |  | 18 |  |
|  |  | 28 |  |
|  |  | 29 |  |
|  |  | 30 |  |
|  |  | 40 |  |
|  |  | 41 |  |
|  |  | 42 |  |
| Comments: |  |  |  |
| Quality Reviewed By/Date: |  |  |  |

APPENDIX 16.4 AUTOCLAVED ITEMS PREPARATION (Cont.)

| Wrapped Items |  |  |  |
| --- | --- | --- | --- |
| Instructions: 1. Each of the 3 operators prepares 8 wrapped items to be autoclaved according to standard procedure, labels each item as specified in the table below, and document the completion. 1.1 Attach the labels directly onto the wrap materials. 1.2 Using a thumb, affix the label onto the pouch. 1.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 2. Record comments and additional information in the “Comments” section or N/A. 3. Set the labeled wrapped items aside upon completion. |  |  |  |
| Instrument Description |  | Label # | Items Prepared and Labeled By (initials and date): |
|  |  | 7 |  |
|  |  | 8 |  |
|  |  | 9 |  |
|  |  | 19 |  |
|  |  | 20 |  |
|  |  | 21 |  |
|  |  | 31 |  |
|  |  | 32 |  |
|  |  | 33 |  |
|  |  | 43 |  |
|  |  | 44 |  |
|  |  | 45 |  |
|  |  | 10 |  |
|  |  | 11 |  |
|  |  | 12 |  |
|  |  | 22 |  |
|  |  | 23 |  |
|  |  | 24 |  |
|  |  | 34 |  |
|  |  | 35 |  |
|  |  | 36 |  |
|  |  | 46 |  |
|  |  | 47 |  |
|  |  | 48 |  |
| Comments: |  |  |  |
| Quality Reviewed By/Date: |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION

| Labels 1- 8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 1 through 8. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 1 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 2 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 3 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 4 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 5 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 6 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 7 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 8 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION (Cont.)

| Labels 9-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 9 through 16. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 9 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 10 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 11 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 12 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 13 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 14 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 15 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 16 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION (Cont.)

| Labels 17-24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 17 through 24. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 17 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 18 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 19 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 20 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 21 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 22 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 23 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 24 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION (Cont.)

| Labels 25-32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 25 through 32. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 25 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 26 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 27 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 28 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 29 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 30 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 31 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 32 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION (Cont.)

| Labels 33-40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 33 through 40. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 33 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 34 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 35 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 36 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 37 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 38 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 39 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 40 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.5 SIMULATED HANDLING & INSPECTION BEFORE STERILIZATION (Cont.)

| Labels 41-48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 41 through 48. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of label on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 41 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 42 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 43 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 44 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 45 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 46 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 47 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 48 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.6 LABEL PRINTING – SET 2

| Group 5 Low/Low Printer Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP5. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#49 – Label#60) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#49 - #60). | Each of the 12 labels contains / does not contain a unique label number (#49 - #60). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.6 LABEL PRINTING – SET 2 (Cont.)

| Group 6 Low/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP6. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#61 – Label#72) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#61 - #72). | Each of the 12 labels contains / does not contain a unique label number (#61 - #72). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.6 LABEL PRINTING – SET 2 (Cont.)

| Group 7- High/Low Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP7. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#73 – Label#84) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#73 - #84). | Each of the 12 labels contains / does not contain a unique label number (#73 - #84). |  |
| 5 | Legibility | All labels are legible | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.6 LABEL PRINTING – SET 2 (Cont.)

| Group 8 - High/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP8. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#85 – Label#96) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#85 - #96). | Each of the 12 labels contains / does not contain a unique label number (#85 - #96). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.7 LABELING BEFORE STERILIZATION

| Pouched Items |  |  |
| --- | --- | --- |
| Instructions: 1. Each of the 3 operators will label 8 pouched packages. 2. Locate packages previously labeled in APPENDIX 16.4 with label numbers as specified. 2.1 Select the location on the package (plastic side) near the existing label. 2.2 Using a thumb, affix another label onto the pouch. 2.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 3. Record comments and additional information in the “Comments” section or N/A. 4. Set the labeled pouched items aside upon completion. |  |  |
| Locate Previously Labeled Package with Label # | Affix Another Label with Label # | New Label Affixed By (initials and date): |
| 1 | 49 |  |
| 2 | 50 |  |
| 3 | 51 |  |
| 13 | 52 |  |
| 14 | 53 |  |
| 15 | 54 |  |
| 25 | 55 |  |
| 26 | 56 |  |
| 27 | 57 |  |
| 37 | 58 |  |
| 38 | 59 |  |
| 39 | 60 |  |
| 4 | 61 |  |
| 5 | 62 |  |
| 6 | 63 |  |
| 16 | 64 |  |
| 17 | 65 |  |
| 18 | 66 |  |
| 28 | 67 |  |
| 29 | 68 |  |
| 30 | 69 |  |
| 40 | 70 |  |
| 41 | 71 |  |
| 42 | 72 |  |
| Comments: |  |  |
| Quality Reviewed By/Date: |  |  |

APPENDIX 16.7 LABELING BEFORE STERILIZATION (Cont.)

| Wrapped Items |  |  |
| --- | --- | --- |
| Instructions: 1. Each of the 3 operators will label 8 wrapped packages. 2. Locate packages previously labeled in APPENDIX 16.4 with label numbers as specified. 2.1 Select the location on the package near the existing label. 2.2 Using a thumb, affix another label onto the pouch. 2.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 3. Record comments and additional information in the “Comments” section or N/A. 4. Set the labeled pouched items aside upon completion. |  |  |
| Locate Previously Labeled Package with Label # | Affix Another Label with Label # | New Label Affixed By (initials and date): |
| 7 | 73 |  |
| 8 | 74 |  |
| 9 | 75 |  |
| 19 | 76 |  |
| 20 | 77 |  |
| 21 | 78 |  |
| 31 | 79 |  |
| 32 | 80 |  |
| 33 | 81 |  |
| 43 | 82 |  |
| 44 | 83 |  |
| 45 | 84 |  |
| 10 | 85 |  |
| 11 | 86 |  |
| 12 | 87 |  |
| 22 | 88 |  |
| 23 | 89 |  |
| 24 | 90 |  |
| 34 | 91 |  |
| 35 | 92 |  |
| 36 | 93 |  |
| 46 | 94 |  |
| 47 | 95 |  |
| 48 | 96 |  |
| Comments: |  |  |
| Quality Reviewed By/Date: |  |  |

APPENDIX 16.8 STERILIZATION

| Gravity Cycle (AUT001) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Locate packages previously labeled in APPENDIX 16.7 with label numbers as specified. 2. Autoclave the packages using the existing Gravity cycle in AUT001. Multiple autoclave runs may be utilized. If a load only contains items for this OQ, Biological Indicators (BIs) are not required. 3. Prepare additional label per APPENDIX 16.9 before a sterilization run is complete. 4. Document sterilization using the Instrument Sterilization Form. 5. Include a copy of the sterilization records with the OQ document. 6. Upon cycle completion, label each package per APPENDIX 16.10. |  |  |  |  |  |  |
| Locate Previously Labeled Package with Label #’s |  | Cycle Type |  | Package Type | Sterilization Load ID | Copy of Sterilization Record Attached? |
| 1 | 49 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 2 | 50 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 3 | 51 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 13 | 52 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 14 | 53 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 15 | 54 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 25 | 55 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 26 | 56 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 27 | 57 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 37 | 58 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 38 | 59 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 39 | 60 | Gravity (AUT001) |  | Pouch |  | Yes / No |
| 7 | 73 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 8 | 74 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 9 | 75 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 19 | 76 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 20 | 77 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 21 | 78 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 31 | 79 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 32 | 80 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 33 | 81 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 43 | 82 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 44 | 83 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| 45 | 84 | Gravity (AUT001) |  | Wrap |  | Yes / No |
| Comments: |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |
| Quality Reviewed By/ Date: |  |  |  |  |  |  |

APPENDIX 16.8 STERILIZATION (Cont.)

| Pre-Vacuum Cycle (AUT002) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Locate packages previously labeled in APPENDIX 16.7 with label numbers as specified. 2. Autoclave the packages using the existing Pre-Vacuum cycle in AUT002. Multiple autoclave runs may be utilized. 3. Prepare additional label per APPENDIX 16.9 before a sterilization run is complete. 4. Document sterilization using the Instrument Sterilization Form. 5. Include a copy of the sterilization records with the OQ document. 6. Upon cycle completion, label each package per APPENDIX 16.10. |  |  |  |  |  |  |
| Locate Previously Labeled Package with Label #’s |  | Cycle Type |  | Package Type | Sterilization Load ID | Copy of Sterilization Record Attached? |
| 4 | 61 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 5 | 62 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 6 | 63 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 16 | 64 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 17 | 65 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 18 | 66 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 28 | 67 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 29 | 68 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 30 | 69 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 40 | 70 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 41 | 71 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 42 | 72 | Pre-Vac (AUT002) |  | Pouch |  | Yes / No |
| 10 | 85 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 11 | 86 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 12 | 87 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 22 | 88 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 23 | 89 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 24 | 90 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 34 | 91 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 35 | 92 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 36 | 93 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 46 | 94 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 47 | 95 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| 48 | 96 | Pre-Vac (AUT002) |  | Wrap |  | Yes / No |
| Comments: |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |
| Quality Reviewed By/ Date: |  |  |  |  |  |  |

APPENDIX 16.9 LABEL PRINTING – SET 3

| Group 9 Low/Low Printer Settings |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP9. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#97 – Label#108) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#97 - #108). | Each of the 12 labels contains / does not contain a unique label number (#97 - #108). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.9 LABEL PRINTING – SET 3 (Cont.)

| Group 10 Low/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 4 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP10. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#109-120 – Label#72) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 4 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#109 - #120). | Each of the 12 labels contains / does not contain a unique label number (#109 - #120). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.9 LABEL PRINTING – SET 3 (Cont.)

| Group 11- High/Low Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 23. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP11. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#121 – Label#132) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 23 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#121 - #132). | Each of the 12 labels contains / does not contain a unique label number (#121 - #132). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.9 LABEL PRINTING – SET 3 (Cont.)

| Group 12 - High/High Printer Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Label Example: Label Example: Instructions: 1. In the printer settings menu, set print speed to 6 and print darkness to 27. Document results. Aktiv Grotesk Medium, bold Size 16 Size 7 Size 10 2. With the ZebraDesigner, open a pre-created label template with file name GROUP12. 2.1 See label example on the right. 2. FILE>PRINT PREVIEW. 3. Enter your initials and enter 12 for quantity. 4. Check and select PRINT. 5. Visually inspect each label for accuracy, ensure each label contains a unique label number, and verified all 12 labels (Label#133 – Label#144) are present. 5.1 If inaccuracy or wrong label number observed, or labels missing, reprint the affected labels. 6. Verify label legibility and document results. 7. Indicate “P” for Pass if the actual results match the expected results. Otherwise, indicate “F” for Fail. 8. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |
| Line # | Item to Verify | Expected Results | Actual Results | Pass (P) / Fail (F) |
| 1 | Print Speed | 6 |  |  |
| 2 | Print Darkness | 27 |  |  |
| 3 | Accuracy and Number of Labels | Correct Initials on all 12 labels | Initials are / are not correct on all 12 labels |  |
| 4 |  | Each of the 12 labels contains a unique label number (#133 - #144). | Each of the 12 labels contains / does not contain a unique label number (#133 - #144). |  |
| 5 | Legibility | All labels are legible. | All labels are / are not legible. |  |
| Comments: |  |  |  |  |
| Executed By/ Date: |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |

APPENDIX 16.10 LABELING AFTER STERILIZATION

| Pouched Items |  |  |  |
| --- | --- | --- | --- |
| Instructions: 1. Each of the 3 operators will label 8 pouched packages IMMEDIATELY AFTER removed from sterilizer & cooled enough to handle. 2. Locate packages previously labeled in APPENDIX 16.7 with label numbers as specified. 2.1 Select the location on the package (plastic side) near the existing label. 2.2 Using a thumb, affix another label onto the pouch. 2.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 3. Record comments and additional information in the “Comments” section or N/A. 4. Set the labeled pouched items aside upon completion. |  |  |  |
| Locate Previously Labeled Package with Label # |  | Affix Another Label with Label # | New Label Affixed By (initials and date): |
| 1 | 49 | 97 |  |
| 2 | 50 | 98 |  |
| 3 | 51 | 99 |  |
| 13 | 52 | 100 |  |
| 14 | 53 | 101 |  |
| 15 | 54 | 102 |  |
| 25 | 55 | 103 |  |
| 26 | 56 | 104 |  |
| 27 | 57 | 105 |  |
| 37 | 58 | 106 |  |
| 38 | 59 | 107 |  |
| 39 | 60 | 108 |  |
| 4 | 61 | 109 |  |
| 5 | 62 | 110 |  |
| 6 | 63 | 111 |  |
| 16 | 64 | 112 |  |
| 17 | 65 | 113 |  |
| 18 | 66 | 114 |  |
| 28 | 67 | 115 |  |
| 29 | 68 | 116 |  |
| 30 | 69 | 117 |  |
| 40 | 70 | 118 |  |
| 41 | 71 | 119 |  |
| 42 | 72 | 120 |  |
| Comments: |  |  |  |
| Quality Reviewed By/Date: |  |  |  |

APPENDIX 16.10 LABELING AFTER STERILIZATION (Cont.)

| Wrapped Items |  |  |  |
| --- | --- | --- | --- |
| Instructions: 1. Each of the 3 operators will label 8 wrapped packages IMMEDIATELY AFTER removed from sterilizer & cooled enough to handle. 2. Locate packages previously labeled in APPENDIX 16.7 with label numbers as specified. 2.1 Select the location on the package near the existing label. 2.2 Using a thumb, affix another label onto the pouch. 2.3 Rub thumb across the label to ensure the label is securely affixed to the pouch. 3. Record comments and additional information in the “Comments” section or N/A. 4. Set the labeled pouched items aside upon completion. |  |  |  |
| Locate Previously Labeled Package with Label # |  | Affix Another Label with Label # | New Label Affixed By (initials and date): |
| 7 | 73 | 121 |  |
| 8 | 74 | 122 |  |
| 9 | 75 | 123 |  |
| 19 | 76 | 124 |  |
| 20 | 77 | 125 |  |
| 21 | 78 | 126 |  |
| 31 | 79 | 127 |  |
| 32 | 80 | 128 |  |
| 33 | 81 | 129 |  |
| 43 | 82 | 130 |  |
| 44 | 83 | 131 |  |
| 45 | 84 | 132 |  |
| 10 | 85 | 133 |  |
| 11 | 86 | 134 |  |
| 12 | 87 | 135 |  |
| 22 | 88 | 136 |  |
| 23 | 89 | 137 |  |
| 24 | 90 | 138 |  |
| 34 | 91 | 139 |  |
| 35 | 92 | 140 |  |
| 36 | 93 | 141 |  |
| 46 | 94 | 142 |  |
| 47 | 95 | 143 |  |
| 48 | 96 | 144 |  |
| Comments: |  |  |  |
| Quality Reviewed By/Date: |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION

| Labels 1- 8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 1 through 8. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 1 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 2 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 3 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 4 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 5 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 6 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 7 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 8 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION(Cont.)

| Labels 9-16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 9 through 16. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 9 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 10 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 11 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 12 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 13 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 14 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 15 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 16 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 17-24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 17 through 24. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 17 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 18 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 19 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 20 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 21 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 22 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 23 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 24 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 25-32 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 25 through 32. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 25 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 26 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 27 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 28 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 29 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 30 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 31 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 32 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 33-40 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 33 through 40. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. . |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 33 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 34 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 35 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 36 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 37 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 38 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 39 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 40 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 41-48 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 41 through 48. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 41 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 42 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 43 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 44 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 45 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 46 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 47 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 48 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 49-56 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 49 through 56. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 49 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 50 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 51 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 52 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 53 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 54 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 55 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 56 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 57-64 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 57 through 64. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 57 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 58 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 59 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 60 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 61 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 62 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 63 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 64 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 65-72 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 65 through 72. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 65 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 66 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 67 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 68 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 69 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 70 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 71 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 72 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 73-80 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 73 through 80. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 73 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 74 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 75 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 76 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 77 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 78 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 79 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 80 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 81-88 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 81 through 88. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 81 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 82 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 83 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 84 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 85 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 86 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 87 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 88 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 89-96 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 89 through 96. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 89 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 90 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 91 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 92 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 93 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 94 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 95 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 96 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 97-104 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 97 through 104. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 97 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 98 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 99 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 100 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 101 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 102 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 103 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 104 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 105-112 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 105 through 112. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 105 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 106 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 107 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 108 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 109 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 110 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 111 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 112 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 113-120 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 113 through 120. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 113 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 114 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 115 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 116 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 117 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 118 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 119 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 120 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 121-128 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 121 through 128. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 121 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 122 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 123 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 124 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 125 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 126 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 127 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 128 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 129-136 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 129 through 136. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 129 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 130 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 131 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 132 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 133 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 134 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 135 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 136 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |

APPENDIX 16.11 SIMULATED HANDLING & INSPECTION AFTER STERILIZATION (Cont.)

| Labels 137-144 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Instructions: 1. Gather pouched and wrapped packages labeled with label numbers 137 through 144. 2. Lightly rub labels 20 times with gloved hand. Indicate “Yes” when complete. 3. Lightly rub labels 20 times with gloved hand, which have been wiped with 70% IPA. Indicate “Yes” when complete. 4. Lightly rub labels 20 times with gloved hand, which have been wiped with prepared Vesphene III. Indicate “Yes” when complete. 5. Lightly rub labels 20 times with gloved hand, which have been wiped with Spor-Klenz.. Indicate “Yes” when complete. 6. Shuffled / rub with each package with 1 or more other package 20 times. Indicate “Yes” when complete. 7. Visually Inspect legibility of each label and verify presence of 3 labels on each package. Document results. 8. Specify P (Pass) if “Yes” is indicated in the results column. 9. Record comments and additional information in the “Comments” section or N/A. |  |  |  |  |  |  |  |  |  |
| Label | 20 x Rubbed with Gloved Hands | 20 x IPA Gloved Hands |  | 20 x Vesphene III Gloved Hands | 20 x Spor-Klenz. Gloved Hands | 20 x Shuffled or rubbed with other items | Results: Label legible? | Results: Label Present? | Pass (P) / Fail (F) |
| 137 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 138 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 139 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 140 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 141 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 142 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 143 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| 144 | Yes / No | Yes / No |  | Yes / No | Yes / No | Yes / No | Yes / No | Yes / No |  |
| Comments: |  |  |  |  |  |  |  |  |  |
| Executed By/ Date: |  |  |  |  |  |  |  |  |  |
| Quality Reviewed By/Date: |  |  |  |  |  |  |  |  |  |